Experimental Validation

The Motor Imagery (MI) BCI FII corpus is used to compare Julia and Python implementations of the Riemannian Minimum Distance to mean (MDM) classifier in term of classification accuracy. The execution time needed in the two implementations is also compared.

Two classification tasks are considered:

1) “right hand” vs. “feet”,

2) “left hand” vs. “right hand”.

Classification Accuracy

The pre-processing and encoding have been carried-out separately in the two languages. For each session in all databases, an 8-fold cross-validation is performed using the StratifiedKFold function of the Python Scikit-learn package to extract the training and validation splits, used identically in both languages.

Pre-processing

Pass-band 4th-order linear-phase response (forward-backward) Butterworth digital filter in the region 8Hz-32Hz.

Encoding

Sample covariance matrix estimation of the MI trials with a Tikhonov regularization (diagonal loading) with parameter 10e-4.

Model fitting and prediction

Riemannian MDM (nearest centroid).

Table 1: Resources for the implementation in Julia and Python

| Implementation | Julia | Python |
| --- | --- | --- |
| Pre-processing | DSP.jl (from Eegle.jl) | SciPy |
| Encoding | Eegle.jl | PyRiemann |
| Model fitting and prediction | PosDefManifoldML.jl | PyRiemann |

Results

1) “right hand” vs “feet”

Table 2: Classification accuracies for “right hand” vs. “feet” MI

| Database | Julia Mean and Std (%) | Python Mean and Std (%) | Difference |
| --- | --- | --- | --- |
| AlexMI-None | 74.99 ± 14.90871 | 75.0 ± 14.9092 | -0.009375 |
| BNCI2014001-None | 84.94 ± 11.90432 | 84.95 ± 11.9096 | -0.011944 |
| BNCI2014002-Test | 78.98 ± 17.0275 | 78.98 ± 17.03021 | -0.001179 |
| BNCI2014002-Train | 76.18 ± 11.3729 | 76.17 ± 11.36971 | 0.005857 |
| BNCI2015001-None | 81.11 ± 12.84454 | 81.1 ± 12.8419 | 0.001214 |
| Schirrmeister2017-None | 86.3 ± 13.98145 | 86.29 ± 13.98154 | 0.005071 |
| Weibo2014-None | 75.66 ± 10.0781 | 75.65 ± 10.07801 | 0.0005 |
| Zhou2016-None | 87.85 ± 5.3869 | 87.84 ± 5.38551 | 0.009591 |

Figure 1: Classification accuracies for “right hand” vs. “feet” MI

2) “left hand” vs. “right hand”.

Table 3: Classification accuracies for left hand vs. right hand MI

| Database | Julia Mean and Std (%) | Python Mean and Std (%) | Difference |
| --- | --- | --- | --- |
| BNCI2014001-None | 78.24 ± 13.80576 | 78.24 ± 13.81513 | -0.004611 |
| BNCI2014004-Test | 76.53 ± 14.12921 | 76.53 ± 14.12962 | 0.000315 |
| BNCI2014004-Train | 64.8 ± 10.11533 | 64.8 ± 10.11401 | 0.002833 |
| Cho2017-None | 62.1 ± 10.29322 | 62.1 ± 10.29349 | -3.8e-5 |
| GrosseWentrup2009-None | 56.72 ± 7.26641 | 56.72 ± 7.26111 | 0.00045 |
| Lee2019_MI-Train | 62.0 ± 13.2963 | 62.0 ± 13.29557 | -0.001285 |
| PhysionetMI-Task 2 | 62.09 ± 12.82857 | 62.09 ± 12.83081 | -0.000216 |
| Schirrmeister2017-None | 70.2 ± 14.54348 | 70.2 ± 14.54554 | -0.002179 |
| Shin2017A-None | 63.25 ± 16.43238 | 63.25 ± 16.43238 | 0.0 |
| Weibo2014-None | 69.14 ± 14.35384 | 69.14 ± 14.35598 | -0.0014 |
| Zhou2016-None | 85.35 ± 7.97634 | 85.35 ± 7.97701 | 0.001682 |

Figure 2: Classification accuracies for left hand vs. right hand MI

Execution Time

The execution time for performing the above cross-validations is estimated as the median execution time observed across five repetitions of the whole cross-validation procedure, excluding pre-processing and encoding. Both in the Julia and Python implementation, multithreading has been enabled, allowing four threads to be used in parallel.

Results

1) “right hand” vs “feet”

Table 4: Execution time for “right hand” vs. “feet” MI

| Database | Julia Mean and Std (ms) | Python Mean and Std (ms) | Difference (abs) |
| --- | --- | --- | --- |
| AlexMI-None | 22 ± 6 | 263 ± 5 | 241 |
| BNCI2014001-None | 123 ± 56 | 355 ± 19 | 232 |
| BNCI2014002-Test | 23 ± 6 | 262 ± 3 | 239 |
| BNCI2014002-Train | 43 ± 13 | 270 ± 8 | 227 |
| BNCI2015001-None | 56 ± 8 | 271 ± 8 | 215 |
| Schirrmeister2017-None | 13125 ± 15535 | 42932 ± 14133 | 29807 |
| Weibo2014-None | 1971 ± 1965 | 3839 ± 2368 | 1869 |
| Zhou2016-None | 40 ± 12 | 270 ± 5 | 230 |

Figure 3: Execution Time (log10) for right hand vs. feet MI

Figure 4: Execution Time (Ratio Python/Julia) for “right hand” vs. “feet” MI

2) “left hand” vs. “right hand”.

Table 5: Execution time for left hand vs. right hand MI

| Database | Julia Mean and Std (ms) | Python Mean and Std (ms) | Difference (abs) |
| --- | --- | --- | --- |
| BNCI2014001-None | 149 ± 65 | 356 ± 23 | 207 |
| BNCI2014004-Test | 14 ± 3 | 264 ± 3 | 249 |
| BNCI2014004-Train | 15 ± 2 | 265 ± 3 | 250 |
| Cho2017-None | 1045 ± 61 | 3101 ± 252 | 2056 |
| GrosseWentrup2009-None | 4021 ± 398 | 20416 ± 2577 | 16395 |
| Lee2019_MI-Train | 613 ± 138 | 2103 ± 256 | 1489 |
| PhysionetMI-Task | 351 ± 279 | 931 ± 204 | 580 |
| Schirrmeister2017-None | 12789 ± 14878 | 42223 ± 13491 | 29435 |
| Shin2017A-None | 25 ± 8 | 269 ± 5 | 243 |
| Weibo2014-None | 1954 ± 1963 | 3791 ± 2301 | 1837 |
| Zhou2016-None | 37 ± 7 | 272 ± 6 | 234 |

Figure 5: Execution Time (log10) for left hand vs. right hand MI

Figure 6: Execution Time (Ratio Python/Julia) for left hand vs. right hand MI

Conclusions

The Julia and Python implementations allows the same accuracy. Typically, the Julia implementation is a few times faster. The speed-up factor with the analyzed data is up to ≈ 25.